Operating System Setup

- Use Raspberry Pi Imager to put DietPi onto the SD card

- In the SD card’s files, edit dietpi.txt to have AUTO_SETUP_NET_WIFI_ENABLED=1

- In the SD card’s files, edit dietpi-wifi to have login credentials to a local wifi network

First Boot

- Set up the Raspberry Pi (with the SD card inside) with its own monitor and let it run until it settles. You should see an IP address

- On your computer, open up cmd and run “ssh root@<the IP you see>” to SSH into the Raspberry Pi

  - The default password is “dietpi”

- Let it run and restart itself, SSH into it again, change passwords (I set them to “secret1225”) and follow instructions to finish installation

Enable USB Peripheral Mode

- Open the config.txt file:

sudo nano /boot/firmware/config.txt

- Add the following to the bottom (on a new line if needed) and save the file:

dtoverlay=dwc2,dr_mode=peripheral

- Open the modules file:

sudo nano /etc/modules

- Add the following and save the file:

dwc2
libcomposite

Create a shared storage space

- Run the following commands (2GB here, change count to change the size)
This will create a 2GB shared storage image file with a FAT32 file system that a connected PC will be able to interface with

sudo mkdir -p /piusb
sudo dd if=/dev/zero of=/piusb/piusb.bin bs=1M count=2000 status=progress
sudo mkfs.vfat -F 32 -n PISHARE /piusb/piusb.bin

Set up a composite USB gadget

- Create the script

sudo nano /usr/local/sbin/usb-gadget.sh

- Enter the following and save the file:

#!/bin/bash
set -e

GADGET=/sys/kernel/config/usb_gadget/pigadget
mkdir -p "$GADGET"
cd "$GADGET"

echo 0x1d6b > idVendor    # Linux Foundation
echo 0x0104 > idProduct   # Multifunction Composite Gadget
echo 0x0100 > bcdDevice
echo 0x0200 > bcdUSB

mkdir -p strings/0x409
echo "0001" > strings/0x409/serialnumber
echo "RaspberryPi" > strings/0x409/manufacturer
echo "Pi4 Serial+Storage Gadget" > strings/0x409/product

mkdir -p configs/c.1/strings/0x409
echo "Serial + Mass Storage" > configs/c.1/strings/0x409/configuration
echo 250 > configs/c.1/MaxPower

# --- Serial (CDC ACM) function ---
mkdir -p functions/acm.GS0
ln -s functions/acm.GS0 configs/c.1/

# --- Mass storage function ---
mkdir -p functions/mass_storage.usb0
echo 0 > functions/mass_storage.usb0/lun.0/cdrom
echo 0 > functions/mass_storage.usb0/lun.0/ro
echo 1 > functions/mass_storage.usb0/lun.0/removable
echo /piusb/piusb.bin > functions/mass_storage.usb0/lun.0/file
ln -s functions/mass_storage.usb0 configs/c.1/

# --- Bind to the actual controller ---
ls /sys/class/udc | head -n1 > UDC

#!/bin/bash
set -e

GADGET=/sys/kernel/config/usb_gadget/pigadget
mkdir -p "$GADGET"
cd "$GADGET"

echo 0x1d6b > idVendor    # Linux Foundation
echo 0x0104 > idProduct   # Multifunction Composite Gadget
echo 0x0100 > bcdDevice
echo 0x0200 > bcdUSB

mkdir -p strings/0x409
echo "0001" > strings/0x409/serialnumber
echo "RaspberryPi" > strings/0x409/manufacturer
echo "Pi4 Serial+Storage Gadget" > strings/0x409/product

mkdir -p configs/c.1/strings/0x409
echo "Serial + Mass Storage" > configs/c.1/strings/0x409/configuration
echo 250 > configs/c.1/MaxPower

# --- Serial (CDC ACM) function ---
mkdir -p functions/acm.GS0
ln -s functions/acm.GS0 configs/c.1/

# --- Mass storage function ---
mkdir -p functions/mass_storage.usb0
echo 0 > functions/mass_storage.usb0/lun.0/cdrom
echo 0 > functions/mass_storage.usb0/lun.0/ro
echo 1 > functions/mass_storage.usb0/lun.0/removable
echo /piusb/piusb.bin > functions/mass_storage.usb0/lun.0/file
ln -s functions/mass_storage.usb0 configs/c.1/

# --- Bind to the actual controller ---
ls /sys/class/udc | head -n1 > UDC

- Compile the above script:

sudo chmod +x /usr/local/sbin/usb-gadget.sh

- Set up a service file so it can run at boot:

sudo nano /etc/systemd/system/usbgadget.service

[Unit]
Description=USB composite gadget (serial + mass storage)
After=local-fs.target

[Service]
Type=oneshot
ExecStart=/usr/local/sbin/usb-gadget.sh
RemainAfterExit=yes

[Install]
WantedBy=sysinit.target

[Unit]
Description=USB composite gadget (serial + mass storage)
After=local-fs.target

[Service]
Type=oneshot
ExecStart=/usr/local/sbin/usb-gadget.sh
RemainAfterExit=yes

[Install]
WantedBy=sysinit.target

sudo systemctl daemon-reload
sudo systemctl enable --now usbgadget.service

Turn on the serial login console

- Run the following command:

sudo systemctl enable --now getty@ttyGS0.service

At this point, you should be able to connect to the Pi’s console and upload files to the Pi just using a USB cable connected to your computer! Now, let’s set up automated script running.

Automated Script Running

- Run the following commands to create some new directories and open up a Python file:

sudo mkdir -p /opt/usb-watcher /opt/payload /mnt/piusb_ro
sudo nano /opt/usb-watcher/watcher.py

- Paste the contents of watcher.py and save the file

- Install pip and dependencies:

sudo apt update && sudo apt install python3-pip
sudo pip3 install inotify_simple --break-system-packages

- Register that file:

sudo chmod +x /opt/usb-watcher/watcher.py

- Set up and run it as a root service:

sudo nano /etc/systemd/system/usb-watcher.service

[Unit]
Description=Watch USB share image and sync into payload dir
After=usbgadget.service
Requires=usbgadget.service

[Service]
Type=simple
ExecStart=/usr/bin/python3 /opt/usb-watcher/watcher.py
Restart=on-failure
User=root

[Install]
WantedBy=multi-user.target

[Unit]
Description=Watch USB share image and sync into payload dir
After=usbgadget.service
Requires=usbgadget.service

[Service]
Type=simple
ExecStart=/usr/bin/python3 /opt/usb-watcher/watcher.py
Restart=on-failure
User=root

[Install]
WantedBy=multi-user.target

sudo systemctl daemon-reload
sudo systemctl enable --now usb-watcher.service
journalctl -u usb-watcher.service -f

Now, whenever you modify the contents of PISHARE, the Pi will automatically look for “main.py” and run it!

Note: pip running on the Pi can’t seem to sustain large package downloads. For larger dependencies, upload them directly to the Pi using PISHARE.

Install some other dependencies needed:

sudo apt install python3.13-venv
sudo apt-get install libportaudio2